Summary of changes

The bilateral filter datapath was rewritten to remove all IEEE floating-point arithmetic from the per-pixel computation. All Gaussian weights (spatial and color) are now precomputed offline and stored as fixed-point lookup tables (weight_t), and the main accumulation uses ap_ufixed types (wsum_t for weight sums and vsum_t for weighted value sums). Per-tap floating-point multiplies were replaced by fixed-point multiplies and additions, and the obsolete floating-point Gaussian data header was removed entirely. This ensures the core convolution loop consists only of fixed-point arithmetic, enabling predictable synthesis and significantly reduced DSP/LUT usage.

The expensive floating-point division used for normalization was replaced with a reciprocal-LUT-based normalization step. The accumulated weight sum is rounded in fixed point, quantized to an index, and used to fetch a precomputed reciprocal (RECIP_LUT), after which normalization is performed via a fixed-point multiply. This eliminates hardware dividers and floating-point units while preserving visually acceptable output. The AXI-stream interface, buffering logic, and output packing remain unchanged, maintaining functional compatibility while meeting the acceptance criteria of no floating-point operators in the synthesized datapath.